Deparment of Computer science and Engineering

Mandsaur University,Mandsure

Computer Architecture And Organazation

Assignment -3

Q.1 An instruction pipeline consists of 4 stages: Fetch (F), Decode field (D), Execute (E), and Result-Write (W). The 5 instructions in a certain instruction sequence need these stages for the different number of clock cycles as shown by the table below.

| Instruction | F | D | E | W |
| --- | --- | --- | --- | --- |
| 1 | 1 | 2 | 1 | 1 |
| 2 | 1 | 2 | 2 | 1 |
| 3 | 2 | 1 | 3 | 2 |
| 4 | 1 | 3 | 2 | 1 |
| 5 | 1 | 2 | 1 | 2 |

Find the number of clock cycles needed to perform the 5 instructions

Q.2 State the Both's algorithm for multiplication of two numbers, Draw a block diagram for the implementation of the Booth's algorithm for determining the product of two 16- bit signed numbers.